TENDER REF: 6300029357

ANNEXURE I – COMPLIANCE TO TECHNICAL SPECIFICATIONS & CONDITIONS

Description  :   :   Supply,  Installation  and  Commissioning of  Portable Electro-Mechanically operated 25 Ton capacity High Lift  Synchronous Jacks( 1 set comprising  of  4  Nos Jacks for synchronized operation).

Qty:  02 SET( 1 set comprising  of  4  Nos Jacks for synchronized operation)

| TECHNICAL INPUTS FOR DESIGN OF THE SYSTEM |  |  | COMPLIED / NOT COMPLIED / REMARKS |
| --- | --- | --- | --- |
| Portable Electro-Mechanically operated 25 Ton capacity High Lift Synchronous Jacks with necessary wheel arrangement for easy movement, with sliding type lifting head (1 set comprising of 4 Nos Jacks for synchronized operation) complete with Drive motor (motor, gear box, brake system) at bottom of the jack frame including 1 No. of central control panel-cum-desk for synchronized group or sub-group operation as well as local push button station for each jack for individual operation, complete with necessary length of PVC power and control cables with plug and socket arrangement for connection between central control panel and jack and extra 10 Mtr length PVC cable for connecting the central panel with main power supply |  |  |  |  |
| 1 | SPECIFICATIONS: |  |  |
| a | LOAD CAPACITY PER JACK | 25 T |  |
| b | DUTY CLASSIFICATION | CLASS-2 |  |
| C | TYPE | PORTABLE |  |
| D | TOTAL LOAD CAPACITY PER SET OF 4 JACKS | 100 T |  |
| E | TYPE OF LIFTING HEAD | SLIDING |  |
| F | LIFTING SPEED | 200 mm/min |  |
| TECHNICAL INPUTS FOR DESIGN OF THE SYSTEM |  |  | COMPLIED / NOT COMPLIED / REMARKS |
| G | MIN CLAW HEIGHT FROM GROUND LEVEL | 650mm(approx) |  |
| H | CLOSED HEIGHT OF LIFTING HEAD | 400 mm |  |
| I | AVAILABLE LIFT | 1750mm (approx) |  |
| J | HEIGHT OF LIFTING HEAD (MAX) | 2400mm (approx) |  |
| K | TOTAL HEIGHT OF JACKS | 3300 mm(approx) |  |
| L | MAXIMUM CLAW PROJECTION | 760 mm(approx) |  |
| M | GROUND CLEARANCE OF NO-LOAD | 20mm(approx) |  |
| N | SIZE OF LIFTING PAD | 250 x 120 mm |  |
| O | BASE PLAN SIZE (a) LENGTH (b) WIDTH | 2000 mm(approx) 940 mm |  |
| P | CROSS TRAVEL OF CLAW | 400 mm |  |
| Q | MOTOR RATING (NOMINAL) | Appropriate capacity. The capacity of motor to be mentioned clearly in the offer. |  |
| R | DESIGN WEIGHT | 2600 kgs (approx.) |  |
| TECHNICAL INPUTS FOR DESIGN OF THE SYSTEM |  |  | COMPLIED / NOT COMPLIED / REMARKS |
| 2 | Brakes: | Firm shall furnish the details of the brake offered in the system.) |  |
| 3 | The following provision in the control panel shall be provided |  |  |
| A | - Control of individual jack from the central control panel & also from individual jacks - Group control/Individual control of jacks with select switch. - Provision of synchronous control for all the jacks by using proximity switch type sensors. |  |  |
| B | The control panel height should be such that it is easy to operate and the same shall be fabricated using thick gauge sheets and shall be sturdy. |  |  |
| C | Load sensor of lifting pad with necessary tell-tale indicator Lamps on control panel |  |  |
| D | Extra gusset projection of 280 x 500 mm on either side of the base width to be provided for more stability. |  |  |
| E | All the electrical components used shall be of BEML approved brand. - (Switch Gears: Siemens / Telemechanic / Schneider) |  |  |
| F | The power and control cables drawn for internal wiring from control board of meters, limit switches etc. should be of ample rating and preferably laid in conduits / metallic hoses / concealed to avoid mechanical damages |  |  |
| G | The power and control circuits shall be properly protected by fuses, MCB's, ELCB's, Overload Relays etc. MPCB to be provided for all motors. |  |  |
| H | Jacks shall be provided with mechanical locking facility in both X & Y movements for safety locking |  |  |
| TECHNICAL INPUTS FOR DESIGN OF THE SYSTEM |  |  | COMPLIED / NOT COMPLIED / REMARKS |
| I | The jacks lifting mechanism shall be provided with suitable pinion extended wheel and engaging arrangement so as to facilitate up/down movement manually in case of any drive problem. |  |  |
| J | The design of the jacks shall be such that in case of power failure the loaded lifting claw shall immediately locked in that position. |  |  |
| K | Chain guard to be provided |  |  |
| L | The motor shall be directly coupled to gear box to reduce abnormal noise during operation. |  |  |
| M | The electrical motor / gear box shall be mounted at bottom of 4the jacks |  |  |
| N | Firm to provide Design validation by an independent government authorized third party for the rated capacity and life cycle of the product for safe operation. |  |  |
| O | Provision of bellows for the load screw at both ends (i.e., top & bottom) |  |  |
| P | Provision for grease nipple for load nut and safety nut. |  |  |
| 4 | Special Safety Features: |  |  |
| A | Once the load limit is sensed the jack should not move upwards in single jack operation. The upward movement should happen only in group operation when all the four load limit switches are sensed. |  |  |
| B | In group operation (Up / Down) synchronous movement of all the four jacks should take place. If any one or two jacks are not operating during group operation, the group operation should not function and there should be a telltale indicator / display to show which are the jacks at fault. |  |  |
| c | Provision of limit switch with fault identification lamp / siren between load nut and safety nut to alert the operators in case of load nut failure and interlock facility such that the jacks will not operate. |  |  |
| TECHNICAL INPUTS FOR DESIGN OF THE SYSTEM |  |  | COMPLIED / NOT COMPLIED / REMARKS |
| d | Display board in all the four jacks regarding do’s and don’ts |  |  |
| e | Regarding safety during lifting / on load jack operations. If there are any failure in the system, the safety nut should hold back ( freeze / jam ) the lifting pad in position without allowing the lifting pad to come down suddenly thereby safety is ensured. |  |  |
| f | The individual set of cables (Power & Control ) shall be cable tied with one mitre distance apart |  |  |
| g | Over the individual set of cable a protective sleeve shall be provided by the vendor to avoid damage and to increase life enhancement. |  |  |
| 5 | Firm should quote separately for one number of load Testing Fixtures for checking/testing the load of individual jacks periodically. |  |  |

Technical Terms & Conditions:

| Sl. No. | Clause | COMPLIED / NOT COMPLIED / REMARKS |
| --- | --- | --- |
| 1 | DELIVERY: To be delivered to BEML, Bangalore Complex. |  |
| 2 | GUARANTEE: The jacks should be guaranteed for a period of 24 calendar months from the date of Commissioning. During this period any part found defective should be repaired / replaced by you at free of cost. |  |
| 3 | COMMISSIONING: Firm should depute their service personnel for installation, commissioning, initial start-up / demonstration of the jacks up to entire satisfaction of BEML at free of cost. It shall be entire responsibility of firm to erect and commission the jacks and demonstrate to entire satisfaction of BEML free of cost. |  |
| 4 | PUBLICATIONS: The following publications should be supplied by you in TRIPLICATE at free of cost well in advance before dispatch of the jacks: - Foundation Drawing - Erection /Commissioning Instruction Manual - Operators Instruction Manual - Manual Maintenance Manual with spare parts catalogue - Electrical circuit diagram, synchronizing unit circuit. - Makers Certificate |  |
| Sl. No. | Clause | COMPLIED / NOT COMPLIED / REMARKS |
| 5 | Firm should submit 3 sets of GA & Electrical Drawings for our approval. Only after receipt of approved Drawings at your end you should take-up manufacturing of the equipment. |  |
| 6 | INSPECTION: The equipment should be offered for inspection by our representatives at your works prior to dispatch for load trial. However, final inspection and acceptance of the equipment will be at BEML's works only. Please inform readiness of the equipment in advance enabling us to depute our representatives for inspection |  |
| 7 | The erection and commissioning of the jacks shall meet the provisions of the Factories Act. All the workmen being employed for erection and commissioning of the jacks should be covered under ESI and necessary safety precautions should be taken including provision of suitable scaffold and safety belts etc as necessary. |  |
| 8 | TRAINING: Firm should impart training to our operators and Maintenance personnel (both mechanical & electrical) at the time of installation / commissioning of the jacks at free of cost. |  |
| Sl. No. | Clause | COMPLIED / NOT COMPLIED / REMARKS |
| 9 | AFTER SALES SERVICE: BEML will not hold any inventory of spares required for maintenance of the jacks. In case of breakdown, you should depute your service personnel within 24 Hours along with necessary spares and set right the jacks. |  |
| 10 | The equipment should be backed-up with quick availability of essential spares both during warranty and out of warranty period and any loss in production / considerable down time during warranty will be at your risk/cost. |  |
| 11 | PROVING TRIALS: During installation of the jacks, Firm should successfully prove the specified parameters to BEML's satisfaction. . |  |
| 12 | The equipment shall be capable to operate on 415 V 10%, 50 Hz 2% AC, 3 Ph supply. |  |
| 13 | All the electrical motors should be tropicalized to suit our existing 415V 10%, 50 Hz 2%, 3 Ph AC power |  |
| Sl. No. | Clause | COMPLIED / NOT COMPLIED / REMARKS |
| 14 | details of the firm for having designed, manufactured, erected, commissioned this type of 16T capacity jack (i.e. each set comprises of 4 Nos., total 64 T cap). or above after 2005 (past 5 years). The details such as complete address, phone/fax/contact person of the user along with year of supply, application to be furnished along with the offer. |  |
| 15 | Deviation Statement: The firm should furnish detailed comparison statement (deviation statement) of our tender enquiry vis-à-vis the equipment offered by the firm for specifications, features and all the points stipulated under the notes of our tender enquiry in the same order of our tender specifications against each of the point. The offer will be ignored/disqualified in case of not furnishing the above technical comparative statement along with technical bid |  |