TENDER REF: 6300029361

PART A – TECHNICAL SPECIFICATIONS & CONDITIONS

Description: WATER CUM FOAM FIRE TENDER as per IS-10460-1983

Qty:      1   No

| Sl no |  |  | Specification |  |  | Remarks |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 |  |  | Chassis | TATA make model: LPT 1613/42 BS IV |  |  |  |
| 02 |  |  | Pump | Single Stage Centrifugal type |  | Firefly CE marked |  |
|  |  |  |  | Capacity 1800 lpm |  |  |  |
|  |  |  |  | Pressure 7kg/cms2 |  |  |  |
| 03 |  |  | PTO (Power Take Off Unit) | Full Torque drive line (Ratio 1:1:27) |  | VAS make |  |
| 04 |  |  | Water Tank | 4500 ltrs |  |  |  |
| 05 |  |  | Foam Tank | 500 ltrs |  |  |  |
| 06 |  |  | Foam Proportioning system | Automatic/Around the pump |  | Ventury type |  |
| 07 |  |  | Mounted equipment | Water/Foam monitor(automatic/manual) |  |  |  |
| 08 |  |  | Supplementary Extinguishers | - C02-22.5kgx4 nos. - DCP 75 kgsx2nos. |  |  |  |
| 09 |  |  | Hose peel | For minimum 60 mtrs hose length |  |  |  |
| 10 |  |  | Drivers’ Cabin | Double compartment type |  |  |  |
| 11 |  |  | Lockers | Shutter/Door type |  |  |  |
| 12 |  |  | Specification/Construction | As Per IS 10460 |  |  |  |
|  | Accessories : The following accessories are to be fitted/provided on the equipment as per relevant IS Standards |  |  |  |  |  |  |  |
|  | Description |  |  |  | Qty. in nos. |  | Remarks |  |
| 1 | Hooter/Siren (Min. 50 watts) |  |  |  | 1 |  |  |  |
| 2 | Hand operated Fire Bell |  |  |  | 1 |  |  |  |
| 3 | Fog Lamps (Halogen min. 12V) |  |  |  | 2 |  |  |  |
| 4 | Spot light (front & back side – one each. Halogen 12 V Min) |  |  |  | 2 |  |  |  |
| 5 | Search light on top of vehicle |  |  |  | 1 |  |  |  |
| 6 | Reverse Lights |  |  |  | 3 |  |  |  |
| 7 | Twin Amber Blinker Lights |  |  |  | 2 |  |  |  |
| 8 | Blinker type Trafficators |  |  |  | 1 |  |  |  |
| 9 | 3 Piece Aluminum Extension ladder min.10.5mtrs (Trussed type) |  |  |  | 1 |  | King make |  |
| 10 | Armored Suction hose 100mm size with round thread coupling to suit the pump inlet min.2.5 mrs fitted with Gun metal couplings on both the side |  |  |  | 4 |  |  |  |
| 11 | Delivery hose (rubber lined) dia 63 mm in 30 mtr lengths. Each length duly fitted with Gun metal instantaneous type couplings make & female on both ends ( Type A ) |  |  |  | 10 |  |  |  |
|  | Description |  |  |  | Qty. in nos. |  | Remarks |  |
| 12 | Gun metal suction strainer |  |  |  | 1 |  | ACB make |  |
| 13 | Basket Strainer (metal) |  |  |  | 1 |  | ACB make |  |
| 14 | Fog nozzle with Extension applicator with log head GM |  |  |  | 1 |  | ACB make |  |
| 15 | Dividing Breaching made of light Alloy |  |  |  | 2 |  | ACB make |  |
| 16 |  | Collecting Breaching made of light Alloy |  |  | 2 |  | ACB make |  |
| 17 |  | Suction Wrenches |  |  | 1 |  | ACB make |  |
| 18 |  | Long line – 50mm circumference 30m long |  |  | 2 |  | Reputed make |  |
| 19 |  | Short line -50mm circumference 15m long |  |  | 2 |  | Reputed make |  |
| 20 |  | Hose bandages – rubberized |  |  | 12 |  | ACB make |  |
| 21 |  | Hose clamps |  |  | 6 |  | ACB make |  |
| 22 |  | Hand controlled branch for 63mm size hose couplings Gunmetal |  |  | 1 |  | ACB make |  |
| 23 |  | Branch pipe universal Gun metal |  |  | 1 |  | ACB make |  |
| 24 |  | Branch with revolving head Gun metal |  |  | 1 |  | ACB make |  |
| 25 |  | Branch pipe Gun metal |  |  | 4 |  | ACB make |  |
| 26 |  | Nozzles of sizes 12mm,16mm,20mm,25mm & 32mm (two each) |  |  | 10 |  | ACB make |  |
| 27 |  | Adapter for 100mm suction female screw coupling and 63mm male instantaneous |  |  | 2 |  | ACB make |  |
| 28 |  | Adapter double male instantaneous pattern 63mm |  |  | 2 |  | ACB make |  |
| 29 |  | Adapter Double female instantaneous pattern 63mm |  |  | 2 |  | ACB make |  |
| 30 |  | Nozzle spanner |  |  | 2 |  | ACB make |  |
| 31 |  | Portable Electric Box lamp with Recharge accumulator |  |  | 2 |  | Reputed make |  |
| 32 |  | Hand lamp |  |  | 2 |  | Reputed make |  |
| 33 |  | Flame Proof lamp |  |  | 2 |  | ACB make |  |
| 34 |  | Rubber Gloves |  |  | 1 |  | Vidyuth make |  |
| 35 |  | Asbestos Gauntlet |  |  | 1 |  | Reputed make |  |
| 36 |  | Axe Large |  |  | 1 |  | Reputed make |  |
| 37 |  | Spade |  |  | 1 |  | Reputed make |  |
| 38 |  | Pick Axe |  |  | 1 |  | Reputed make |  |
| 39 |  | Crow Bar |  |  | 1 |  | Reputed make |  |
| 40 |  | Sledge Hammer 6.5kg |  |  | 1 |  | Reputed make |  |
| 41 |  | Hydraulic /Pneumatic Jack 7.5 T |  |  | 1 |  | Reputed make |  |
| 42 |  | Carpenter Saw 60 cm |  |  | 1 |  | Reputed make |  |
| 43 |  | Fire Hook |  |  | 1 |  | ACB make |  |
| 44 |  | Tool Kit (firm to indicate the full details) |  |  | 1 set |  | Reputed make |  |
| 45 |  | First Aid Box (equipped with adequate medicines) for minimum 10 persons |  |  | 1 |  |  |  |
| 46 |  | Nylon Rope 16mm dia, 30m long |  |  | 2 |  |  |  |
| 47 |  | Manila Rope 20mm dia, 30m long |  |  | 1 |  |  |  |
| 48 |  | AFF Foam Jerry cans (20 ltrs) |  |  | 2 |  | AFFF 3% ; ISI |  |
| 49 |  | Hand held Torches ( Rechargeable Type ) |  |  | 2 |  |  |  |
| 50 |  | Dust masks |  |  | 12 |  |  |  |
| 51 |  | Leather Gloves (full size) |  |  | 6 pairs |  |  |  |
| 52 |  | Shock Resistant Gloves (full size) |  |  | 5 pairs |  |  |  |
| 53 |  | PVC Gloves (full size) |  |  | 5 pairs |  |  |  |
| 54 |  | Proximity suit ( Protective clothing for fire men complete with gloves, boots & helmets with face shield made of material capable of reflecting at least 95% radiant heat around 1500 – 2000 degree Celsius and also afford some protection against direct flame. The suit to be of sufficient size for extra large size users and should be able to accommodate a breathing apparatus |  |  | 2 Sets. |  | Modern Apparel make |  |
| 55 |  | Self contained Breathing apparatus with Spare cylinder with tools |  |  | 1 set |  | Vijay Sabre make |  |

|  | INSTRUMENTS & CONTROLS |  |  |  |
| --- | --- | --- | --- | --- |
|  | The control panel at the rear should include the following : |  |  |  |
| 01 | Compound Gauge |  |  |  |
| 02 | Pressure Gauge |  |  |  |
| 03 | Engine Throttle Control |  |  |  |
| 04 | Temperature Gauge for Engine Temperature |  |  |  |
| 05 | Pump Hour meter |  |  |  |
| 06 | Control valve for water tank to Primer |  |  |  |
| 07 | 5 Way control valve |  |  |  |
| 08 | Water level indicator |  |  |  |
| 09 | A bell to and from the driver cabin |  |  |  |
| 10 | Necessary valves at appropriate locations to control the functions – Firm should indicate details of valves like type, qty. & location & purpose |  |  |  |
|  | DESIGN AND CONSTRUCTION |  |  |  |
| 1 | Chassis – TATA Model LPT 1613/42 BS IV complying with latest pollution norms |  |  |  |
| 2 | PUMP - A single stage centrifugal type fire pump of 1800 ltrs per minute @ 7g/cm2 capacity should be provided at the rear of the tender. - Pump body shall be gun metal - Pump shaft shall be of stainless steel - Impeller neck ring shall be manufactured from high quality bronze - Impeller must be dynamically balanced and shall be made of gun metal - Pump should be provided with mechanical carbon seal, self adjusting type - Suction inlet shall be provided with 100 mm suction hose connection - Delivery outlet shall be provided with 2 nos x 63 mm delivery valves with female instantaneous couplings and blank cap. - A stainless steel drain plug shall be provided at the bottom of casing. - Pump should be fitted with automated water ring primer capable of lifting water to 7.3m at NTP condition |  | Firefly CE marked |  |
| 3 | POWER TAKE OFF UNIT : Heavy duty power take-off unit transmitting adequate power to the water pump should be fitted behind the engine gear box to drive the pump at the rear. Lever of the Power take-off unit should be fitted in the drivers compartment |  | VAS make |  |
| 4 | WATER TANK: The water tank of 4500 litres capacity is to be fitted on the chassis on heavy bearers. The tank is to be fabricated out of suitable thickness MS plates (bottom plate not less than 5mm thick, 3.15mm side & Top MS Plates) with adequate baffles and mounted behind the cab so as to allow full contents to flow into the Pump. - One inspection manhole with a cover at the top and a cleaning hole of suitable diameter with cover at the base should be provided. - All bolting should be done with stainless steel nuts & bolts. The tank should be so designed and mounted as to the centre of gravity of the appliance as low as is possible. The mounting should permit full contents of the tank to flow into the pump. - The tank should be coated internally & externally with anti corrosive treatment - Should be provided with overflow pipes - The inlet of the filling pipe should be situated at the rear of the appliance and is to be fitted with 63mm delivery hose male instantaneous hydrant connection - The tank should be connected to the Pump and hose reel and control valves should be provided in such a way that all the following operations are possible : Hydrant to tank - Hydrant to Reel - Tank-Pump Reel - Hydrant Pump Reel - Pump Tank - Off |  |  |  |
| 5 | FOAM TANK : - A compound tank of min 500 ltrs capacity should be mounted on the chassis - Should be a separate and distinct unit which can be removed for replacement - Should be of rigid type, made of stainless steel welded construction - Should withstand a hydrostatic pressure of 0.3kg/cm2 - Should have filling orifice with a removable stainless steel strainer - Draw off tube should be connected to the foam compound proportionator/inductor and pump - Hand operated foam transfer pump made of gun metal 20lpm cap should be fitted in a locker for filling the foam along with stainless steel plumbing with ball valve arrangement - Hose should be made of PVC spiral type The Pickup tube connection should be at Pump control panel. Tube type foam level indicator of acrylic tube should also be provided at the pump control panel and calibrated as empty. |  |  |
|  | NOTE : This applicable to Sl. No. 4 & Sl. No. 5 Tank Mounting System Subframe & Metacone System : The water & foam tanks is to be mounted on the vehicle on a subframe using Rubber Metacones. This subframe to be made from Anti-Corrosive Treated MS 4” Section and be bolted with the chassis using the high tensile bolts. ‘U’ Bolts shall not be used for mounting of tanks on vehicle. The rubber metacones shall facilitate to absorb the jerks and bending torsions in expansion as well as compression mode without thigh defection. Tank should be mounted on the chassis in a manner keeping in view the proper load distribution on the axles. The baffles be arranged in a manner to facilitate easy cleaning of the tanks. The tank should be mounted on two / three cross bearers to counteract stresses caused by chassis flexing. The Centre of Gravity shall be maintained as low as possible. |  |  |
| 6 | FOAM COMPOUND PROPORTIONATOR/INDUCTOR Around the Pump, a proportionator should be fitted between the suction and the delivery of the pump which will induct foam compound into the water stream with no loss in delivery pressure from the pump. Should comprise of inductor and selector valve Should have “OFF” position so as to use the equipment as Water Tender | ACB make |  |
| 7 | FOAM MONITOR - A foam monitor should be mounted on the roof of locker just behind the cab such that it can be operated manually - Should be capable of traversing through 360 Deg. In Horizontal plane and elevating from horizontal to 45 deg. & depressing from horizontal to not less than 15 deg and fully rotating in both the directions - Should be self aspiring type - Should be capable of projecting the foam discharge to an effective distance not less than 35mtrs in still air when operated at 7kg/cm2 - Monitor assembly should be of light aluminum alloy material with locking system | ACB make |  |
| 8 | SUPPLEMENTARY EXTINGUISHING AGENT - C02 Equipment : 4 nos., of C02 Cylinders each of 22.5kg should be mounted on the appliance. These are to be provided in a battery of 2 nos., on each side with one manifold for each set of two cylinders - The cylinders should be fitted with puncture type valves. Tenderer should provide 2 nos., of hose reels for C02 discharge. Each reel to be fitted with 15 mtrs long high pressure hose terminating into a C02 horn - The C02 Extinguisher should be provided in lockers at a convenient position on the appliance - Dry chemical Powder – 2 nos., detachable mobile type dry powder extinguisher capacity 75kg each fixed at a convenient position for easy removal and taking it to the fire spot | ISI marked CCE / PESO Approved ISI marked |  |
| 9 | HOSE REEL : One Hose reel should be fitted with 60 mtrs of 20mm bore hose and a shut-off nozzle is to be provided on the appliance in an accessible position and that has to work through the fire pump of appliance | ISI marked |  |
| 10 | COOLING SYSTEM : Indirect cooling system of open circuit type consisting of special heat exchanger should be provided on the vehicle to enable full power output to be maintained during pumping duty without overheating and hot water is to be discharged to waste |  |  |
| 11 | DRIVERS CABIN : The cabin should be closed type with double compartments without partition, fabricated to accommodate 1 driver and one shift in-charge in front two seats and a crew of five Firemen in the rear / second row of seats. The cabin should have four doors, the seats should be properly cushioned and upholstered with good quality Rexene. The body shall be made of minimum 2mm thick MS Pressed sections, square tubes, angles etc. The complete structure shall be first ZINC PLATED and then EPOXY painted for better corrosion resistance. The body shall be externally paneled with min. 1.6mm aluminium sheets and internally paneled with minimum 1.2mm aluminium sheets. Min. 25mm thick Thermocol should be sandwiched between internal & external panels. Flooring shall be of 2mm thick aluminium chequered plate. The doors on both sides should have sliding type aluminium panel windows. The aluminium panel should be powder coated. On both the sides of driver, rear view mirrors should be of toughened glass type. |  |  |
|  | Driver Seat shall be adjustable |  |  |
|  | Doors to be fitted with safety Glasses and winding type Regulators |  |  |
|  | One roof light is to be provided in the compartment |  |  |
|  | Plastic coated grab rails and non slip steps shall be provided wherever necessary |  |  |
|  | An electric bell shall be provided in the drivers cabin to send signals from rear pump controlled panel to the driver |  |  |
| 12 | LOCKERS : Lockers on both the sides of the body with Aluminium shutters of German design and closing type are to be provided for the storage of equipments. Main equipment lockers are to be provided with automatic lights inside. |  |  |
| 13 | GALLOWS FOR LADDER : Gallows to carry 10.5 mtr alloy aluminium extension ladder should be provided on the appliance |  |  |
| 14 | PAINTING/MARKING: The appliance should be painted in PO RED with service lettering/marking, driving compartment and inside lockers with grey paint, chassis and wheels are to be in black colour, distinctive colours for water and foam lines. Two coats of anti-corrosion and primary coat before painting. |  |  |

TERMS & CONDITIONS  :

- Delivery:  Vehicle should be registered and to be delivered at BEML, Bangalore Complex.  Guarantee / Warranty Card / Standard Free Service Card / Coupon to be provided at the time of delivery of the Vehicle.

- The foam tender including all the accessories should be designed, manufactured, tested etc., as per relevant Indian, International Standards, TACs requirements and as per Sound Engineering practice

- All the equipment and accessories should be fixed on the appliance in a compact and neat manner and should be so placed that each part is easily and readily accessible for use and maintenance.  The center of gravity should be kept as low as possible.

- The firm should commission the tender at our works and should prove the satisfactory performance of the same at no extra cost and on turn-key-basis with all system on ready to use stage.

- The equipment should be guaranteed for a minimum period of 24 months from the date of commissioning against defective design, material, workmanship & performance

- Performance Bank Guarantee:  PBG to be executed for 10% of the order value covering the warranty period.

- Publications like Operation Instruction Manual, Maintenance manual, wiring diagram, Spare Parts manual, Detailed drawings, Charts containing exploded view of Sub-assemblies etc., should be supplied in printed form as well as on CD Rom.  Three sets along with the equipment

- Other items like Service Tool Kit – one set to be supplied

- RTO:   It is the entire responsibility of the firm to commission the vehicle and submit Registration Certificate (Smart Card) with Comprehensive Insurance, Life Time Tax Receipt and to be registered at Bengaluru.

- Training:  Necessary training to our personnel to be imparted at no extra cost and should cover the following:

- Operation of equipment

- Assembly/Dismantling of Pump & Primer

- Trouble shooting

# Compliance to Technical specifications and Technical terms & conditions as per Annexure I [Mandatory]should be uploaded in SRM portal in the Technical bid.