TENDER REF: 6300029365

PART A – TECHNICAL SPECIFICATIONS & CONDITIONS

DESCRIPTION:    Supply, Erection, Integration, Installation, Testing  &  Proving  of  CLOSED CIRCUIT TELEVISION Camera System (Phase-II)    AND   also  ensuring the full functioning of existing  CCTV  System at BEML LTD, Bangalore complex

ELIGIBILITY CRITERIA :  The Bidders should submit the following documents along with the Technical bid.

- The firms should submit all the relevant supporting documents like Purchase Order copies and Performance Reports from end users for having supplied, commissioned and proven similar system.

- Only OEM and their Authorized Dealers are eligible to quote. The authorized Dealers should submit duly valid authorization letter.

- Audited Financial report of the firm for the last 3-Financial years should be provided.

DETAILS OF EXISTING CCTV SYSTEM AT BEML  : Presently, BEML Bangalore Complex is having All-in-One CCTV Server of “Hikvision Blazer Pro” Model No-128/16H which is already integrated with 85 no’s of cameras.

SCOPE OF WORK :

- Includes supply, integration and installation of 38-Nos of New Cameras, Installation of 5-Nos of New Cameras already available with BEML, removing 6-Nos of Existing Cameras from their present locations and Re-fixing the same to new locations as per Camera Location table I, II & III and also ensuring full functioning of the existing CCTV System.

- Supply and installation of network accessories including Civil Works, Commissioning of all the cameras by integrating with the existing CCTV Server available at Factory Main Gate security office, Testing, Proving and Handing over of the entire CCTV System including the above cameras on Turn-Key basis.

- All cameras and accessories supplied & installed should be fully compatible with the existing CCTV System &

software (“Hikvision Blazer Pro” Model No-128/16H).

- Firms to attend the pre-bid meeting at BEML Ltd., Bangalore Complex to study the existing CCTV System and the location of Cameras, etc., and requirement in detail before participating in the tender.

- Firms should use Pipes and Conduits to lay the cables.

- The firms should provide cable duct to lay cable in order to facilitate future repairs / maintenance works etc., while laying the cable duct work the firm should take care not to disturb the existing water, sanitary line, storm water drain, electrical and telephone cables etc. (the firms may contact our Civil Construction and Electrical Maintenance Department In-charge during their visit before quoting).

- The firms should arrange to remove debris / muck as and when the work is progressing.

- As per ISO 14001 and 18001, the equipment and its connected accessories should be eco-friendly and the equipment’s are well tested before installation.

- The firms should ensure necessary safety precautions for their workmen while carrying out the work at heights or near power lines.

- Firms has to supply and lay power cables for cameras in required places.

- CIVIL WORKS: Firms should also quote for the civil works pertaining to the laying of cables, trench cutting on soil and back filling & welding brackets for Camera fixation, if any.

- The firm / vendor has to ensure the functioning of the entire CCTV System (including existing 85 + 5 Nos of cameras and 38 Nos of newly installed cameras) for 12 months.  After 12-months the firm / vendor has to be ready for AMC  of the entire CCTV System  at BEML Ltd., Bangalore Complex at the discretion of BEML.

- Only the materials / spares cost towards repairing / functioning of existing 85 + 5 Nos of cameras will be borne by BEML for the period of 12 months.

SPECIFICATIONS: DETAILS OF CAMERAS REQUIRED: Mandatory

| Sl. No. | Camera Description | Specifications | Qty. |
| --- | --- | --- | --- |
| 1 | Fixed Outdoor Bullet Camera | IP Camera; Full HD 1920 X 1080 or higher; IR 50 yds or higher; 0.01 Lux or better; Line-crossing video Analytics; (ONVIF) Compliant or Equivalent; H.264 Compression or better. The firm / vendor should provide Make & Model number of the Cameras. | 25 Nos |
| 2 | Fixed Indoor Dome Camera | IP Camera; Full HD 1920 X 1080 or higher; IR 20 yds or higher; 0.01 Lux or better; Line-crossing video Analytics; (ONVIF) Compliant or Equivalent; H.264 Compression or better. The firm / vendor should provide Make & Model number of the Cameras. | 13 Nos |

DETAILS OF ACCESSORIES ESTIMATED : Informative

| Sl No | Description of Accessories | Estimated Qty | Remarks |
| --- | --- | --- | --- |
| 1 | OFC Cable – Armored 6-Core Single Mode | 1800-mtrs Approx. | Firms should provide per unit price for all accessories and per mtr cost for all cables in the commercial bid. Payment will be made for actual usage of accessories and cables. |
| 2 | GI Pipe 1.5-inch internal diameter with accessories | 1000-mtrs Approx. |  |
| 3 | CAT 6 Armored Cable with laying and conduits | 2500-mtrs Approx. |  |
| 4 | POE Switch with racks 8 ports GIGA BYTE POE Switch 10/100/1000Mbps | 9-nos Approx. |  |
| 5 | 1KV UPS with Weather proof Rack and fittings | 5-nos |  |
| 6 | Camera Pole-Galvanized MS 20-feet length (with brackets/angles etc.) | 20-nos Approx. |  |
| 7 | Power supply cables along with Conduits 3 core 1.5Sqmm Cable | 2000 mtrs |  |
| 8 | Installation & Commissioning | - Lot |  |
| 9 | Media convertor along with standard accessories | 12 pairs |  |
| 10 | Laying Charges for OFC cable | 2800 M |  |
| 11 | CAT 6 Unarmored Cable with laying and conduits | 1000 M |  |
| 12 | Water Proof outdoor racks for POE | 5 Nos |  |
| 13 | HDPE Pipe | 1000 M |  |
| 14 | Digging | 1000 M |  |
| 15 | Soil Digging and concrete digging per meter 1 feet depth | 1000 M |  |

(NOTE: The above accessories are not exhaustive, and any other miscellaneous accessories that may be required will be deemed to have been included under “Installation & Commissioning”).

DETAILS OF ACCESSORIES AVAILABLE WITH BEML : The following new items / accessories are available at BEML Stores and these are to be used for installation of the tendered CCTV Cameras and its networking. Bidders are required to inspect the items/accessories available at BEML Stores when they come for pre-bid meeting at BEML Ltd Bangalore Complex, and submit their quotations by considering the usage of these items for the project, which will be provided by BEML during execution of the project.

| Sl. No. | Description of Accessories available | Qty. |
| --- | --- | --- |
| 1 | 15 U Rack | 01 Nos |
| 2 | Networking Server Racks | 03 Nos |
| 3 | 28-Port POE Switch | 02 Nos |
| 4 | 08-Port POE Switch | 01 Nos |
| 5 | 6TB Hard Disks | 04 Nos |
| 6 | Media Converters 10/100 BASE Tx | 02 Nos |
| 7 | Armored 6-Core OFC Single Mode cable | 1000 mtrs Approx. |

- At  BEML  Factory   :   Specification of cameras to be supplied and installed in the locations given below. (Informative)

| Sl. No. | Location/Areas/ Accessory Type | Camera/ Accessory Description | Specifications | Qty. | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | At vulnerable zones along perimeter wall | Fixed Outdoor Bullet Camera | Full HD 1920 X 1080 or higher; IR 50yds or higher; 0.01 Lux or better; Line crossing video Analytic with provision of alarm; (ONVIF) Compliant or Equivalent (H.264 Compression). | 21 Nos |  |
| 2 | Admin Reception | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 3 | Senior Exe-Canteen Corridor | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 4 | RK Office Corridor | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 5 | Kannada Sangha Car Parking | Fixed Outdoor Bullet Camera | Full HD 1920 X 1080 or higher; IR 50yds or higher; 0.01 Lux or better; Line crossing video Analytic with provision of alarm; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 6 | R&D Corridor | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No | Existing Camera to be re-fixed at Kalamandira |
| 7 | Purchase Room | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No | Existing Camera to be re-fixed at Kalamandira |
| 8 | IP Gate Inside Room | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 9 | Unloading Ramp at Goliath Yard |  | Existing Camera : 30m IR water proof Bullet Network Camera |  | To be fixed with existing Camera (IP Gate Inside Room) |
| 10 | Visitor Room | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 11 | Unloading Ramp at near N1 Hangar |  | Existing Camera : 30m IR water proof Bullet Network Camera |  | To be fixed with existing Camera (Visitor Room) |
| 12 | Project Store Office | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 13 | Admin Car Parking adjacent to Kennel Rly Tracks |  | Existing Camera : 30m IR water proof Bullet Network Camera |  | To be fixed with existing Camera (Project Store Office) |
| 14 | Admin Block Visitors’ car parking area (Covering North) | Fixed Outdoor Bullet Camera | Full HD 1920 X 1080 or higher; IR 50yds or higher; 0.01 Lux or better; Line crossing video Analytic with provision of alarm; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 15 | Admin Gate Room | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 16 | Admin Block Visitors’ car parking area (Covering South) |  | Existing Camera : 30m IR water proof Bullet Network Camera |  | To be fixed with existing Camera (Admin Gate Room) |

- At BEML Medical Centre (OPD): Specification of cameras with location and Qty. (Existing OFC LAN connection from EDP office to OPD one pair of optic line to be used for connection with Existing CCTV Server at Factory Main Gate) - (Informative)

| 1 | T-Junction of Pharmacy corridor and RH Office corridor (Covering | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| --- | --- | --- | --- | --- | --- |
| 2 | T-Junction of Pharmacy corridor and RH Office corridor (Covering X-Ray Room) |  | Existing Camera : 30m IR water proof Bullet Network Camera |  | From existing Stock at Stores |
| 3 | Pharmacy & Card Counter | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 4 | Consultation Waiting room | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 5 | Janata Bazar Pharmacy | Fixed Indoor Dome Camera | Full HD 1920 X 1080 or higher; 0.01 Lux or better.; IR 20 yards or higher; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| 6 | Right side near Pharmacy window (Covering portico and Main Gate Gate) | Fixed Outdoor Bullet Camera | Existing Camera : 30m IR water proof Bullet Network Camera |  | From existing Stock at Stores |
| 7 | Left side near Casualty (covering Casualty portico and backside) | -do- | Existing Camera : 30m IR water proof Bullet Network Camera |  | From existing Stock at Stores |
| 8 | Back of OPD Near Back-Gate (covering CMO Office side) | -do- | Existing Camera : 30m IR water proof Bullet Network Camera |  | From existing Stock at Stores |
| 9 | Back of OPD Near Back-Gate (one covering parking shed) | -do | Existing Camera : 30m IR water proof Bullet Network Camera |  | From existing Stock at Stores |

- At  BEML  KALAMANDIRA  (KM)  : Specification of cameras with Location and Qty. (New underground OFC line to be drawn to connect these 4 Nos of cameras upto Medical Centre (OPD).   From OPD to existing CCTV Server at Factory Main Gate via the existing  OFC  LAN Connection) - (Informative)

| 1 | Near Store Room under Staircase (Focusing towards Gate/Road) | Fixed Outdoor Bullet Camera | Full HD 1920 X 1080 or higher; IR 50yds or higher; 0.01 Lux or better; Line crossing video Analytic with provision of alarm; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| --- | --- | --- | --- | --- | --- |
| 2 | Behind KM Right Side (Focusing towards Water Tank) |  | Existing Camera : 30m IR water proof Bullet Network Camera |  | Refix from Purchase Room |
| 3 | Catering Gate (Focusing towards Water Tank) |  | Existing Camera : 30m IR water proof Bullet Network Camera |  | Refix from R&D Corridor |
| 4 | Behind KM Right Side (Focusing towards Mandir) | Fixed Outdoor Bullet Camera | Full HD 1920 X 1080 or higher; IR 50yds or higher; 0.01 Lux or better; Line crossing video Analytic with provision of alarm; (ONVIF) Compliant or Equivalent (H.264 Compression). | 1 No |  |
| Total |  |  |  | 38 Nos |  |

- Display of Main Gate, Administrative Gate and Salvage yard Camera images to be connection to the Personal Computer available at our Chief of Complex Chamber. (Informative)

NOTES: (Informative)

- Firm has to visit BEML to study the requirement in detail and should quote for all the necessary items required for the job.

- Delivery : The firm should supply, install, commission, prove and handover the system within 4 weeks from the date of P.O.

- Guarantee: The 38-Cameras along with all accessories / spares supplied by the firm / vendor shall be guaranteed for a period of 12 calendar months from the date of commissioning. During this period any part / accessory found defective shall be repaired / replaced by the firm at free of cost.

NOTE: For the existing CCTV System, the firm / vendor also has to ensure its full functioning during the period of 12 calendar months.  For the existing CCTV System BEML will bear the materials / spares cost towards repairing / functioning of existing 85 + 5 no’s of cameras.

- Commissioning: The firm should depute their service personnel crew for the installation & commissioning. It should be the entire responsibility of the firm to supply, Erect, install and commission, prove and handing over the entire system to the satisfaction of User Dept.  BEML will only provide electricity, water, Material Handling Facility required for the installation.  Rest of the things to be arranged by the firm.

The erection & commissioning should meet the provision of factories act.

All the workmen being employed for erection & commissioning should be covered under ESI &  necessary safety precautions should be taken compulsorily by the firm.

- The offers should be complete with:

- Detailed technical specifications along with complete construction details

- Sketch/catalogue/schematic diagram.

- Complete scope of work

- List of Accessories

- The quotation should be complete along with List of Accessories.

- Publication: The firm should furnish following publications in triplicate along with the offer.

- Operation instruction manual

- Detailed spare parts catalogue.

- Electric Wiring Diagram

- Schematic Diagram of Cabling.

- Training : The firm should arrange the training for the operation and maintenance to BEML personnel at our works up to entire satisfaction of BEML at free of cost.

- Firm should furnish comparison statement of the offer vis-a-vis our tender enquiry.  BEML will sought Technical Clarification if required during Technical scrutiny for informative clauses only.

- Service backup:

- In case of breakdown during/after warranty the firm should attend with necessary spares within 24 hours of working days and set right the equipment.

- The firm should furnish the service backup network available with them for attending the breakdowns within 24 hours of working days effectively.

- The firm should have service centre at Bangalore.  Address to be provided in Technical Bid.

- The firm should execute the work on TURN-KEY basis.  Any civil / cable laying work / mechanical work required during installation of the  system  has to be carried out by the supplier as TURN-KEY Project.

- Compliance to Technical specifications and Technical terms & conditions as per Annexure I [mandatory] should be uploaded in SRM portal in the Technical bid.

- Firm should Provide unit price for all the items like  cameras,  accessories  per pieces,  cables per meter, Civil works per meters,  installation  and commissioning  in  commercial  bid. L1 will be arrived on total project cost

# Compliance to Technical specifications and Technical terms & conditions as per Annexure I [Mandatory]should be uploaded in SRM portal in the Technical bid.